IoT-Based Pothole and Speed-Hump Detection with On-Device Machine Learning and an AWS Serverless GIS Map

A CAPSTONE PROJECT REPORT

submitted in partial fulfilment of the requirements for the course

CLOUD COMPUTING

by

[STUDENT NAME]  ([REGISTER NUMBER])

Under the guidance of

[GUIDE NAME], [DESIGNATION]

[DEPARTMENT / SCHOOL]

[INSTITUTION NAME, CITY]

October 2026

BONAFIDE CERTIFICATE

This is to certify that the capstone project report entitled “IoT-Based Pothole and Speed-Hump Detection with On-Device Machine Learning and an AWS Serverless GIS Map” is a bonafide record of the work carried out by [STUDENT NAME] ([REGISTER NUMBER]) under my supervision, in partial fulfilment of the requirements for the course Cloud Computing. The contents of this report have not been submitted elsewhere for the award of any degree or diploma.

| [GUIDE NAME] Project Guide | [HEAD OF DEPARTMENT] Head of the Department |
| --- | --- |
| Internal Examiner | External Examiner |

DECLARATION

I hereby declare that the capstone project report entitled “IoT-Based Pothole and Speed-Hump Detection with On-Device Machine Learning and an AWS Serverless GIS Map” submitted for the course Cloud Computing is a record of original work done by me under the guidance of [GUIDE NAME]. The hardware, firmware, machine-learning pipeline and cloud deployment described here were designed, built and tested as part of this project. Sources of external information are acknowledged in the references.

| Place: [CITY] Date: [DATE] | [STUDENT NAME] |
| --- | --- |

ACKNOWLEDGEMENT

I express my sincere gratitude to my project guide, [GUIDE NAME], for the guidance and encouragement throughout this project, and to the Head of the Department and the faculty of [DEPARTMENT] for the facilities and support provided. I also thank my friends and family for their help with testing and for their constant support.

[STUDENT NAME]

ABSTRACT

Potholes damage vehicles and cause accidents, yet road agencies typically learn about them late and through manual complaints. This project presents an end-to-end, low-cost system that detects potholes and speed humps automatically from a moving vehicle and publishes them to a shared, live map in the cloud. An ESP32 microcontroller reads a three-axis accelerometer (MPU-6050 or ADXL345) at 100 Hz and a u-blox NEO-7M GPS receiver at 5 Hz. An orientation-independent event detector tracks the gravity vector, extracts the vertical dynamic acceleration, and cuts a 1.5 s window around every jolt larger than 0.25 g. Twenty shape features computed from the window are classified on the device by a 10-tree random forest of only 17 KB, which distinguishes potholes (drop first) from speed humps (lift first) and from ordinary disturbances.

Each pothole or speed hump is sent with the GPS position captured at the instant of impact as a ~350-byte JSON message over MQTT with mutual-TLS X.509 authentication to AWS IoT Core. A fully serverless backend, described as infrastructure-as-code with AWS SAM, routes messages through an IoT rule to an AWS Lambda function that validates them, stores the raw report in Amazon DynamoDB and folds it into a geohash map cell with atomic counters, so that repeated reports of the same pothole by any number of vehicles become a single marker whose report count expresses confidence. A second Lambda behind an Amazon API Gateway HTTP API serves a Leaflet/OpenStreetMap web map and a standard GeoJSON feed usable in QGIS.

Under five-fold session-grouped cross-validation on 1 625 events from 58 sessions, the deployed model reaches a macro-F1 of 0.88 (pothole F1 0.90, speed-hump F1 0.90); on held-out real motions never used in any form it labels 91 % of drop-then-lift events as potholes and 80 % of lift-then-drop events as speed humps. The complete chain from sensor to map was deployed in the AWS ap-south-1 region and verified with the real device: all six reports produced were stored and mapped, about two seconds after the trigger, at a running cost within the AWS Free Tier.

Keywords: pothole detection, Internet of Things, edge machine learning, random forest, ESP32, MQTT, AWS IoT Core, AWS Lambda, DynamoDB, serverless computing, GIS, GeoJSON.

TABLE OF CONTENTS

ABSTRACT	v

LIST OF FIGURES	viii

LIST OF TABLES	ix

LIST OF ABBREVIATIONS	x

1  INTRODUCTION	1

1.1  Background	1

1.2  Problem Statement	1

1.3  Objectives	1

1.4  Scope	2

1.5  Organisation of the Report	2

2  LITERATURE SURVEY	3

2.1  Vehicle-Based Road-Surface Monitoring	3

2.2  Machine Learning for Road Events	3

2.3  Edge Computing and Serverless Cloud	3

2.4  Geospatial Representation	4

2.5  Gap Addressed	4

3  SYSTEM REQUIREMENTS AND TOOLS	5

3.1  Functional Requirements	5

3.2  Non-Functional Requirements	5

3.3  Hardware	5

3.4  Software and Services	6

4  SYSTEM DESIGN	7

4.1  Overall Architecture	7

4.2  Edge Design	7

4.3  Message Contract	9

4.4  Cloud Design	9

4.5  Security Design	10

5  IMPLEMENTATION	11

5.1  Firmware	11

5.2  Machine-Learning Pipeline	11

5.3  Cloud Deployment	12

5.4  Web Map	13

6  RESULTS AND DISCUSSION	14

6.1  Feature Design	14

6.2  Model Selection	15

6.3  End-to-End Verification	17

6.4  Cost	17

6.5  Problems Encountered	18

6.6  Limitations	18

7  CONCLUSION AND FUTURE WORK	19

7.1  Conclusion	19

7.2  Future Work	19

REFERENCES	20

APPENDIX A: IOT POLICY AND IOT RULE	21

APPENDIX B: REPOSITORY STRUCTURE	21

LIST OF FIGURES

Figure 1:  System architecture: edge classification and serverless cloud	7

Figure 2:  Vertical dynamic acceleration around the trigger: a real speed hump (lift first) and the mirrored pothole training example (drop first)	8

Figure 3:  Macro-F1 of axis-statistics (v1) versus shape (v2) features on simulated data	14

Figure 4:  Cross-validated macro-F1 of all 17 candidate models	15

Figure 5:  Confusion matrix of forest_n10_d8 (five-fold session-grouped CV)	16

Figure 6:  Ten most important features of the deployed forest	16

LIST OF TABLES

Table 1:  Hardware components	5

Table 2:  Software, libraries and cloud services	6

Table 3:  The 20 features computed on the device	8

Table 4:  AWS services and their roles	9

Table 5:  Machine-learning pipeline stages	11

Table 6:  Cross-validated per-class performance of the deployed model	15

Table 7:  Reports received from the device (coordinates omitted)	17

Table 8:  Verification summary	17

Table 9:  Problems encountered and their solutions	18

LIST OF ABBREVIATIONS

| API | Application Programming Interface |
| --- | --- |
| AWS | Amazon Web Services |
| CA | Certificate Authority |
| CV | Cross-Validation |
| GIS | Geographic Information System |
| GNSS / GPS | Global Navigation Satellite System / Global Positioning System |
| HDOP | Horizontal Dilution of Precision |
| HTTP(S) | Hypertext Transfer Protocol (Secure) |
| I2C | Inter-Integrated Circuit bus |
| IAM | AWS Identity and Access Management |
| IaC | Infrastructure as Code |
| IoT | Internet of Things |
| JSON | JavaScript Object Notation |
| MQTT | Message Queuing Telemetry Transport |
| NMEA | National Marine Electronics Association (GPS sentence format) |
| OSM | OpenStreetMap |
| QoS | Quality of Service |
| RTOS | Real-Time Operating System |
| SAM | AWS Serverless Application Model |
| TLS | Transport Layer Security |
| TTL | Time To Live |
| UART | Universal Asynchronous Receiver-Transmitter |
| UBX | u-blox binary protocol |

CHAPTER 1

INTRODUCTION

1.1  Background

Road-surface defects are a persistent safety and maintenance problem, particularly on roads with heavy traffic and seasonal rain. Potholes damage tyres, rims and suspensions, cause sudden braking and swerving, and are a recurring cause of two-wheeler accidents. Road agencies usually learn about potholes through manual inspection or citizen complaints, which is slow, incomplete and difficult to prioritise.

Every vehicle that drives over a pothole experiences a characteristic vertical acceleration. Low-cost MEMS accelerometers, GPS receivers and Wi-Fi-capable microcontrollers make it possible to turn ordinary vehicles into mobile road sensors, and managed cloud IoT services make it possible to aggregate their reports into a shared map without operating any servers. This project combines both: machine learning at the edge to recognise road events, and a serverless cloud backend to collect, aggregate and visualise them.

1.2  Problem Statement

Design and implement a low-cost system that (i) detects potholes automatically while driving, (ii) distinguishes potholes from speed humps, which produce similarly large vertical jolts, (iii) attaches an accurate geographic position to every detection, and (iv) publishes detections from any number of vehicles to a cloud backend that aggregates repeated reports into a live, shareable map.

1.3  Objectives

1.  Build a vehicle-mounted device from an ESP32, a three-axis accelerometer and a GPS receiver.

2.  Develop an event detector that works at any mounting angle and ignores flat road.

3.  Train a compact classifier that runs on the microcontroller and separates pothole, speed hump and normal events.

4.  Publish classified events with GPS position securely over MQTT/TLS to AWS IoT Core.

5.  Deploy a serverless backend (IoT rule, Lambda, DynamoDB, API Gateway) as infrastructure-as-code.

6.  Aggregate reports from many vehicles into map markers and serve them as a web map and a GeoJSON feed.

7.  Verify the complete chain end-to-end with the real device.

1.4  Scope

The project covers the hardware prototype, three ESP-IDF firmware applications (data collector, bench inference, IoT publisher), a Python machine-learning pipeline (collection, simulation, augmentation, training, C export) and an AWS backend with a web map. Model training uses a car-suspension simulator and hand-made bench recordings; collecting and validating on real road data is identified as future work.

1.5  Organisation of the Report

Chapter 2 reviews related work. Chapter 3 lists the requirements and tools. Chapter 4 presents the system design and Chapter 5 its implementation on the device, in the machine-learning pipeline and in the cloud. Chapter 6 reports the results and discusses problems encountered, and Chapter 7 concludes with future work.

CHAPTER 2

LITERATURE SURVEY

2.1  Vehicle-Based Road-Surface Monitoring

Eriksson et al. introduced Pothole Patrol [1], in which taxis equipped with accelerometers and GPS detected potholes using a cascade of hand-tuned filters (speed, high-pass, z-peak, x-z ratio, speed-versus-z ratio) and clustered detections across vehicles to suppress false positives. Their key observation, that repeated reports at the same location increase confidence, is used directly in the aggregation of this project.

Mohan et al. presented Nericell [2], which used smartphone accelerometers, microphones and GPS to monitor braking, bumps and honking, and introduced virtual re-orientation of an arbitrarily placed accelerometer using the gravity vector. Mednis et al. [3] compared threshold-based algorithms (Z-THRESH, Z-DIFF, STDEV(Z), G-ZERO) for real-time pothole detection on Android phones and reported true-positive rates of up to about 90 %, while noting the sensitivity of fixed thresholds to vehicle and mounting.

2.2  Machine Learning for Road Events

Later work replaced hand-tuned thresholds by supervised classifiers on windowed features. Tree ensembles such as random forests [4] and extremely randomised trees [5] are attractive for embedded deployment: they need no feature scaling, handle heterogeneous features, provide feature importances and compile to a few kilobytes of comparisons. This project uses scikit-learn [6] to train and select among such models and exports the chosen forest to C.

2.3  Edge Computing and Serverless Cloud

Edge computing moves computation close to where data is produced to reduce bandwidth and latency [7]. Classifying road events on the device reduces the uplink from about 1 KB/s of raw samples per vehicle to a few hundred bytes per event. On the cloud side, serverless computing [8] (functions as a service and managed databases billed per request) suits the bursty, low-volume traffic of a road-monitoring fleet. MQTT [9] is the dominant lightweight publish/subscribe protocol for IoT devices, and AWS IoT Core [10] provides a managed MQTT broker with per-device X.509 authentication and a rules engine.

2.4  Geospatial Representation

GeoJSON [11] is the standard open format for exchanging geographic features over the web and is read by desktop GIS (QGIS) and web libraries such as Leaflet [12]. Geohash [13] encodes latitude and longitude into a string whose prefix identifies a rectangular cell, which makes it a natural partition key for spatial aggregation in a key-value store.

2.5  Gap Addressed

Earlier systems either used fixed thresholds that depend on vehicle and mounting, ran on smartphones, or stored data on self-managed servers. This project combines an orientation-independent detector, a small learned classifier running on a microcontroller, and a fully managed serverless backend with crowd aggregation and open GIS output, all deployable with a single infrastructure-as-code template.

CHAPTER 3

SYSTEM REQUIREMENTS AND TOOLS

3.1  Functional Requirements

•  Detect vertical jolts while driving and classify each as pothole, speed hump or normal.

•  Record the GPS position at the moment of impact.

•  Publish potholes and speed humps (not normal events) with position, confidence and severity.

•  Keep working without network coverage and send queued events after reconnection.

•  Store every report; show aggregated potholes and humps on a public web map and as GeoJSON.

•  Filter the map by event type and minimum number of reports.

3.2  Non-Functional Requirements

•  Low cost and low power: a single microcontroller with no external compute.

•  Security: mutual-TLS device authentication; devices may only publish to their topic.

•  Scalability without servers to manage; costs proportional to usage.

•  Reproducibility: the entire backend described as code and deployable with one command.

3.3  Hardware

Table 1: Hardware components

| Component | Specification | Interface / pins |
| --- | --- | --- |
| ESP32 DevKit V1 (ESP32-WROOM-32) | 2 × Xtensa LX6 240 MHz, 520 KB RAM, Wi-Fi 2.4 GHz | USB power |
| MPU-6050 (GY-521) — or ADXL345 (GY-291) | 3-axis accelerometer, ±8 g, 100 Hz, auto-detected | I2C: SDA GPIO21, SCL GPIO22 |
| u-blox NEO-7M (GY-NEO7M) | GPS, configured to 5 Hz, GGA + RMC | UART2: RX GPIO16, TX GPIO17 |
| On-board LED | GPS / publish status | GPIO2 |
| Phone hotspot | 2.4 GHz Wi-Fi uplink | — |

3.4  Software and Services

Table 2: Software, libraries and cloud services

| Area | Tools |
| --- | --- |
| Firmware | ESP-IDF v6.1, FreeRTOS, esp-mqtt 1.1, mbedTLS, xtensa-esp-elf GCC 15.2, esptool |
| Machine learning | Python 3, NumPy, SciPy, pandas, scikit-learn, pyserial, matplotlib, pytest, ctypes |
| Cloud | AWS IoT Core, AWS Lambda (Python 3.12, arm64), Amazon DynamoDB, Amazon API Gateway (HTTP API), AWS CloudFormation, Amazon S3, Amazon CloudWatch Logs, AWS IAM |
| Cloud tooling | AWS CLI, AWS SAM CLI 1.169, boto3 |
| GIS | Leaflet 1.9, OpenStreetMap tiles, GeoJSON, geohash; QGIS / kepler.gl compatible |
| Engineering | Git and GitHub, Markdown and Mermaid documentation |

CHAPTER 4

SYSTEM DESIGN

4.1  Overall Architecture

Figure 1 shows the architecture. The edge device performs sensing, detection and classification and sends only classified events. The cloud side consists exclusively of managed AWS services in the ap-south-1 (Mumbai) region: IoT Core receives MQTT messages, an IoT rule forwards them to an ingest Lambda function that writes to DynamoDB, and an HTTP API with a second Lambda function serves the map and the GeoJSON feed.

Figure 1: System architecture: edge classification and serverless cloud

4.2  Edge Design

4.2.1  Orientation-independent event detection

Every accelerometer sample a (in g) updates a slow low-pass estimate of gravity g (time constant about 1 s, adapting ten times more slowly during an event). The vertical dynamic acceleration is v = a·ĝ − |g| and the horizontal component is h = |a − g − v ĝ|. An event is triggered when |v| exceeds 0.25 g; the window contains 0.5 s before and 1.0 s after the trigger (150 samples at 100 Hz), which covers the run-up and both axles at normal speeds, followed by a 0.3 s cooldown. Flat road never triggers and is therefore never classified, which suppresses false positives and keeps the CPU almost idle.

Figure 2: Vertical dynamic acceleration around the trigger: a real speed hump (lift first) and the mirrored pothole training example (drop first)

4.2.2  Features

Twenty features summarise each window (Table 3). Unlike per-axis statistics, they are independent of the mounting angle and capture the order and shape of the excursion, which is what separates a pothole (the wheel first drops, then hits the far edge) from a speed hump (the wheel is first lifted).

Table 3: The 20 features computed on the device

| # | Features | Purpose |
| --- | --- | --- |
| 0–4 | v_max, v_min, v_p2p, v_std, v_mean | size of the event |
| 5 | first_peak | sign of the first large excursion (drop first = pothole) |
| 6 | t_max_minus_t_min | order of the extremes |
| 7 | active_time | duration above threshold |
| 8–10 | jerk_max, hf_ratio, crest | sharp edges, high-frequency energy, spikiness |
| 11–13 | h_max, h_std, h_v_ratio | roll: single-wheel potholes vs. full-width humps |
| 14 | zcr | oscillation rate |
| 15 | pre_rms | roughness of the road just before the event |
| 16–17 | neg_area, pos_area | amount of downward vs. upward motion |
| 18–19 | skew, kurtosis | asymmetry and peakedness |

4.2.3  Firmware tasks

The firmware runs four FreeRTOS tasks: a high-priority sensor task pinned to core 1 that reads the accelerometer, runs the detector and classifier and enqueues alerts; a publish task that sends queued alerts when MQTT is connected (an alert leaves the 32-entry queue only after a successful publish); an LED/status task; and the GPS task that parses NMEA sentences in the background. The GPS state is copied at the trigger instant, so the reported position is where the wheel hit, not where the car was after the 1 s window.

4.3  Message Contract

Potholes and speed humps with a class probability of at least 0.6 are published to topic pothole/alerts with QoS 1. Without a fresh GPS fix (older than 3 s) the coordinates are null.

{"device_id":"pothole-6acecc","event":"POTHOLE","confidence":0.75,"severity_g":0.98, "coordinates":[<lat>,<lon>],"lat":<lat>,"lon":<lon>, "gps":{"fix":true,"sats":7,"hdop":2.4,"speed_kmh":0.5,"age_ms":1167}, "utc":"2026-10-21T21:38:06.800Z","uptime_ms":74668,"model":"forest_n10_d8"}

4.4  Cloud Design

Table 4: AWS services and their roles

| Service | Resource | Role |
| --- | --- | --- |
| AWS IoT Core | Thing, X.509 certificate, policy | device identity, mutual-TLS authentication, publish-only authorisation |
| AWS IoT Core | Topic rule | SELECT *, timestamp() AS received_ms, clientid() AS clientid FROM 'pothole/alerts' → Lambda |
| AWS Lambda | IngestFunction | validate, store raw report, aggregate into geohash cell |
| AWS Lambda | ApiFunction | serve the HTML map and the GeoJSON feed |
| Amazon DynamoDB | PotholeTable (on-demand, TTL) | raw reports (90 days) and aggregated map cells |
| Amazon API Gateway | HTTP API, $default stage | GET / and GET /potholes over HTTPS |
| AWS IAM | two execution roles | CRUD for ingest, read-only for the API |
| AWS CloudFormation / SAM | stack pothole-map | deploy, update and delete all of the above as one unit |
| Amazon S3, CloudWatch Logs | SAM artefact bucket, log groups | deployment packages; one log line per message |

4.4.1  Data model

A single DynamoDB table holds two item types. Raw reports use the partition key REPORT#<device_id> and the receive time as sort key and expire after 90 days via TTL. Map cells use CELL#<geohash-8> and TYPE#POTHOLE or TYPE#SPEED_HUMP; they hold running sums of latitude, longitude and confidence, a report count, the set of reporting devices, the maximum severity and first/last-seen times. Updates use atomic ADD expressions, so concurrent reports from many vehicles need no read-modify-write cycle.

4.4.2  Aggregation and validation

A report is mapped only if its event is POTHOLE or SPEED_HUMP, its coordinates are present, valid and not (0, 0), and its HDOP is at most 5; otherwise it is stored with the reason it was skipped. A geohash-8 cell measures about 38 m × 19 m. At query time the API merges cells of the same type whose centroids lie within 15 m (a pothole on a cell border), weighting by report count, and returns a GeoJSON FeatureCollection with the number of reports and devices, mean confidence, maximum severity and timestamps of each marker.

4.5  Security Design

•  Each device has its own X.509 certificate; the private key never leaves the device and is git-ignored.

•  The IoT policy only allows iot:Connect and iot:Publish to topic pothole/alerts; the device cannot subscribe or read.

•  The ingest Lambda has no public endpoint; only the IoT rule can invoke it.

•  Lambda roles follow least privilege (read/write for ingest, read-only for the API).

•  The map API is public and read-only; it exposes positions and timestamps, not credentials.

CHAPTER 5

IMPLEMENTATION

5.1  Firmware

Three ESP-IDF projects share drivers in a common components/ directory: accel (I2C driver that auto-detects an ADXL345 or MPU-6050 and delivers sample-exact 100 Hz data; the MPU-6050 is polled on its DATA_RDY flag because its byte FIFO was found to desynchronise), neo7m_gps (UART reader, NMEA RMC/GGA parser with checksum, UBX configuration to 5 Hz), road_event (detector and features) and pothole_model (the generated classifier header). esp32_collector streams raw samples for training, esp32_inference prints every classified event for bench tests, and esp32_iot is the deployed application. The application image is about 0.95 MB, so the single-app-large partition table is used.

The road_event C source is pure C99 without ESP-IDF dependencies. The same file is compiled into a shared library on the development computer and called from Python through ctypes, so the features used for training are bit-for-bit those computed on the device.

5.2  Machine-Learning Pipeline

Table 5: Machine-learning pipeline stages

| Script | Output | Function |
| --- | --- | --- |
| ml/collect.py | data/<label>/.csv, .marks | record labelled sessions over serial; guided mode prompts each repetition |
| ml/simulate.py | data/synthetic | quarter-car model: random car, speed, mounting angle, ISO 8608 roughness [14], joints, manholes |
| ml/augment.py | data/bench_aug | rotation, amplitude 0.7–1.35×, time stretch 0.85–1.2×, fresh noise |
| ml/dataset.py | events + features | runs the firmware's C detector via ctypes; drops implausible data |
| ml/train.py | pothole_model.h, model/report.md | model search, session-grouped CV, C export and check |

Data. The deployed model was trained on 1 625 events from 58 sessions (normal 398, speed hump 660, pothole 567): simulated drives, augmented bench recordings and the real bench recordings themselves. The real data are hand motions with the MPU-6050 mounted upright (two normal sessions with 15 events and two speed-hump sessions with 87 clean lift-first events after 42 failed attempts were discarded). Because no clean real pothole was recorded, pothole examples are the real humps mirrored vertically plus parametric drop–jolt–ringing events. Before augmentation, real events are filtered by their defining shape.

Model selection. Seventeen candidates were evaluated: decision trees (depth 3–8), random forests and extremely randomised trees (10 or 25 trees, depth 4–8), all with balanced class weights. Each was scored by macro-F1 under five-fold session-grouped cross-validation, so that no recording session appears in both training and test folds. The smallest model within 0.01 of the best score is selected.

Export. The forest is written as flat node arrays (feature index, threshold, children) in flash, with leaf class probabilities quantised to uint8. The exporter verifies that the generated C code reproduces scikit-learn's predictions on every training event.

5.3  Cloud Deployment

5.3.1  Device provisioning

A script, cloud/provision_device.sh, registers the device in the AWS account with the AWS CLI: it creates the IoT thing pothole-esp32 and the policy PotholeDevicePolicy, generates an active key pair and certificate, attaches policy and thing to the certificate, downloads Amazon Root CA 1, and writes the account's ATS data endpoint into the firmware's git-ignored secrets.h. The firmware is then rebuilt and flashed; no firmware source changes are needed.

cloud/provision_device.sh ap-south-1 pothole-esp32 cd esp32_iot && idf.py build && idf.py -p /dev/ttyUSB0 flash monitor

5.3.2  Infrastructure as code

The backend is a single AWS SAM template (cloud/template.yaml) defining the table, both functions with their IAM policies, the IoT rule and the HTTP API. The first deployment was created as a CloudFormation change set, reviewed (eleven resources, all additions) and executed; updates are a single command.

cd cloud sam validate --lint && sam build sam deploy --stack-name pothole-map --region ap-south-1 --resolve-s3 --capabilities CAPABILITY_IAM

5.3.3  Lambda functions

Both functions are in cloud/src/potholes.py (Python 3.12, boto3 only). ingest_handler validates the message, writes the raw report, computes the geohash and performs one atomic UpdateItem with ADD for the counters and sums and a conditional update for the maximum severity. api_handler returns the map page for GET / and, for GET /potholes, scans the cell items, merges neighbours and returns GeoJSON, with optional type and min_reports filters.

5.4  Web Map

The map page uses Leaflet with OpenStreetMap tiles and needs no API key. It polls the GeoJSON feed every 30 s, draws circle markers whose radius grows with the report count (red for potholes, blue for speed humps), shows report count, devices, confidence, severity and timestamps in a popup with a Google Maps link, and offers filters by type and minimum report count. Potholes are drawn after speed humps so that nearby hump markers never hide them.

CHAPTER 6

RESULTS AND DISCUSSION

6.1  Feature Design

On simulated drives with random mounting angles, the original pipeline that used per-axis minimum, maximum and mean values reached a macro-F1 of only 0.20 with a decision tree and 0.28 with a random forest, because axis statistics mostly encode the mounting. The orientation-independent shape features raise this to 0.84 and 0.92 respectively (Figure 3).

Figure 3: Macro-F1 of axis-statistics (v1) versus shape (v2) features on simulated data

6.2  Model Selection

Figure 4: Cross-validated macro-F1 of all 17 candidate models

The 25-tree depth-8 forest scored highest (0.881) but needs 3 769 nodes. The 10-tree depth-8 forest scored 0.877 with 1 452 nodes (about 17 KB of flash) and was selected; its C export agrees with scikit-learn on 100 % of events and classifies a window in microseconds.

Table 6: Cross-validated per-class performance of the deployed model

| Class | Precision | Recall | F1-score | Support |
| --- | --- | --- | --- | --- |
| normal | 0.83 | 0.84 | 0.83 | 398 |
| speed hump | 0.91 | 0.89 | 0.90 | 660 |
| pothole | 0.89 | 0.91 | 0.90 | 567 |
| macro average | 0.88 | 0.88 | 0.88 | 1 625 |

Figure 5: Confusion matrix of forest_n10_d8 (five-fold session-grouped CV)

The most important features are t_max_minus_t_min (0.187) and first_peak (0.130), the order of the extremes and the sign of the first excursion, followed by skew and hf_ratio (Figure 6). This confirms the physical intuition that potholes and humps differ mainly in which direction the wheel moves first.

Figure 6: Ten most important features of the deployed forest

A leak-free check augmented from only half of the real humps and tested on the other half, which was never seen in any form: 80 % of the unseen lift-then-drop motions were classified as speed humps and 91 % of the same motions mirrored were classified as potholes. On the device, pothole, speed-hump and normal desk motions are all recognised, and no false events occur at rest.

6.3  End-to-End Verification

The backend was deployed to ap-south-1 and the device was provisioned and flashed. With the device on a phone hotspot and a GPS fix of 7–8 satellites, the serial log showed MQTT connected to AWS IoT within 14 s of boot. Table 7 lists the reports received from the device; all were stored and mapped.

Table 7: Reports received from the device (coordinates omitted)

| UTC (2026-10-21) | Event | Confidence | Severity | Satellites / HDOP |
| --- | --- | --- | --- | --- |
| 21:34:14 | SPEED_HUMP | 0.72 | 0.79 g | 7 / 2.4 |
| 21:37:43 | SPEED_HUMP | 0.78 | 0.89 g | 8 / 0.9 |
| 21:38:06 | POTHOLE | 0.75 | 0.98 g | 7 / 2.4 |
| 21:40:07 | SPEED_HUMP | 0.71 | 0.99 g | fix |
| 21:40:34 | POTHOLE | 0.73 | 0.66 g | fix |
| 21:41:09 | POTHOLE | 0.64 | 0.80 g | fix |

The three pothole reports, a few metres apart, were merged by the API into a single marker with reports: 3. The cloud path was also tested independently by publishing a synthetic message with the AWS CLI, and the Lambda logic by an offline unit test with an in-memory table. The latency from trigger to database is about 1.5–2 s, dominated by the 1 s post-trigger window.

Table 8: Verification summary

| Test | Method | Result |
| --- | --- | --- |
| Lambda logic | offline unit test, in-memory table | passed |
| Template | sam validate --lint | valid |
| Cloud path | synthetic publish via aws iot-data publish | marker appeared in GeoJSON |
| Map page | HTTP GET | 200 OK |
| Full system | real device, real GPS fix | 6 / 6 reports mapped; 3 potholes → 1 marker |

6.4  Cost

For a handful of devices all services stay within the AWS Free Tier: IoT Core messaging, Lambda invocations, DynamoDB on-demand requests, HTTP API requests, a few kilobytes in S3 and CloudWatch Logs. The expected monthly cost of the prototype is effectively zero, and costs grow linearly with the number of events rather than with provisioned capacity.

6.5  Problems Encountered

Table 9: Problems encountered and their solutions

| Problem | Cause | Solution |
| --- | --- | --- |
| v1 labels did not match the signal | recorder read stale serial data; windows not aligned to the event | event-aligned windows; guided collection |
| Sessions of random ±8 g values | MPU-6050 byte FIFO desynchronised | read each sample on DATA_RDY |
| False speed humps at rest | MPU-6050 returns 0 g as first sample after reset | fast gravity convergence during warm-up |
| Pothole never predicted | 35 % of hump labels were failed drop-first attempts | shape filtering; potholes from mirrored humps |
| Device could not reach AWS | firmware still had another account's endpoint, certificate and Wi-Fi | provision_device.sh, new secrets.h, reflash |
| Potholes appeared missing on the map | a nearby hump marker was drawn on top at the default zoom | draw potholes last; allow closer auto-zoom |

6.6  Limitations

•  The model has seen simulated car physics and hand motions only; real-road accuracy remains to be measured.

•  No clean real pothole recordings exist; pothole examples are mirrored humps and parametric events.

•  GPS error of a few metres is comparable to pothole size; the 15 m merge radius may join very close potholes.

•  The public map has no authentication and no way to mark potholes as repaired.

CHAPTER 7

CONCLUSION AND FUTURE WORK

7.1  Conclusion

The project delivers a complete, working pipeline from an accelerometer in a vehicle to a pin on a shared web map. An orientation-independent detector and twenty shape features allow a 17 KB random forest on an ESP32 to separate potholes from speed humps with a cross-validated macro-F1 of 0.88. Events are published securely over MQTT/TLS to AWS IoT Core, and a serverless backend defined in one SAM template stores, aggregates and serves them as a Leaflet map and as open GeoJSON. The deployment was verified end-to-end with the real device at negligible cost, showing that edge machine learning combined with managed cloud services is a practical basis for crowd-sourced road monitoring.

7.2  Future Work

•  Record guided and free drives in real vehicles and retrain on road data; evaluate per-speed accuracy.

•  Add an authenticated endpoint to mark potholes as fixed and age out old reports.

•  Use AWS IoT device shadows and OTA updates to distribute retrained models to the fleet.

•  Evaluate a small 1-D convolutional network (TensorFlow Lite Micro with ESP-NN) once several hundred real events per class exist.

•  Replace the bootstrap root access key by an IAM user and narrow the IoT policy to the device's client ID.

REFERENCES

[1]  J. Eriksson, L. Girod, B. Hull, R. Newton, S. Madden, and H. Balakrishnan, “The Pothole Patrol: Using a mobile sensor network for road surface monitoring,” in Proc. 6th Int. Conf. Mobile Systems, Applications, and Services (MobiSys), 2008, pp. 29–39.

[2]  P. Mohan, V. N. Padmanabhan, and R. Ramjee, “Nericell: Rich monitoring of road and traffic conditions using mobile smartphones,” in Proc. 6th ACM Conf. Embedded Network Sensor Systems (SenSys), 2008, pp. 323–336.

[3]  A. Mednis, G. Strazdins, R. Zviedris, G. Kanonirs, and L. Selavo, “Real time pothole detection using Android smartphones with accelerometers,” in Proc. Int. Conf. Distributed Computing in Sensor Systems and Workshops (DCOSS), 2011, pp. 1–6.

[4]  L. Breiman, “Random forests,” Machine Learning, vol. 45, no. 1, pp. 5–32, 2001.

[5]  P. Geurts, D. Ernst, and L. Wehenkel, “Extremely randomized trees,” Machine Learning, vol. 63, no. 1, pp. 3–42, 2006.

[6]  F. Pedregosa et al., “Scikit-learn: Machine learning in Python,” J. Machine Learning Research, vol. 12, pp. 2825–2830, 2011.

[7]  W. Shi, J. Cao, Q. Zhang, Y. Li, and L. Xu, “Edge computing: Vision and challenges,” IEEE Internet of Things Journal, vol. 3, no. 5, pp. 637–646, 2016.

[8]  E. Jonas et al., “Cloud programming simplified: A Berkeley view on serverless computing,” Univ. California, Berkeley, Tech. Rep. UCB/EECS-2019-3, 2019.

[9]  OASIS, “MQTT Version 3.1.1,” OASIS Standard, Oct. 2014.

[10]  Amazon Web Services, “AWS IoT Core Developer Guide.” [Online]. Available: https://docs.aws.amazon.com/iot/latest/developerguide/

[11]  H. Butler, M. Daly, A. Doyle, S. Gillies, S. Hagen, and T. Schaub, “The GeoJSON format,” IETF RFC 7946, Aug. 2016.

[12]  V. Agafonkin, “Leaflet: An open-source JavaScript library for mobile-friendly interactive maps.” [Online]. Available: https://leafletjs.com/

[13]  G. Niemeyer, “Geohash,” 2008. [Online]. Available: http://geohash.org/

[14]  ISO 8608:2016, Mechanical vibration — Road surface profiles — Reporting of measured data. International Organization for Standardization, 2016.

[15]  Espressif Systems, “ESP-IDF Programming Guide.” [Online]. Available: https://docs.espressif.com/projects/esp-idf/

[16]  Amazon Web Services, “AWS Serverless Application Model Developer Guide.” [Online]. Available: https://docs.aws.amazon.com/serverless-application-model/

APPENDIX A: IOT POLICY AND IOT RULE

{"Version": "2012-10-17", "Statement": [ {"Effect": "Allow", "Action": "iot:Connect", "Resource": "arn:aws:iot:ap-south-1:<account>:client/*"}, {"Effect": "Allow", "Action": "iot:Publish", "Resource": "arn:aws:iot:ap-south-1:<account>:topic/pothole/alerts"}]}

SELECT *, timestamp() AS received_ms, clientid() AS clientid FROM 'pothole/alerts'

APPENDIX B: REPOSITORY STRUCTURE

components/ shared ESP-IDF components: accel, neo7m_gps, road_event, pothole_model, board esp32_collector/ firmware: stream raw data for training esp32_inference/ firmware: offline bench test esp32_iot/ firmware: detection + GPS + AWS IoT MQTT ml/ collect, review, dataset, simulate, augment, train, tests cloud/ SAM template, Lambda code, map.html, provision_device.sh, tests model/report.md latest training report docs/ documentation, figures, presentation, reports